- Scope.

This specification documents the detail requirements for space qualified product manufactured on Analog Devices, Inc.’s QML certified line per MIL-PRF-38535 Level V except as modified herein.  The manufacturing flow described in the STANDARD SPACE LEVEL PRODUCTS PROGRAM brochure is to be considered a part of this specification. This brochure may be found at: 	http://www.analog.com/aerospace.  This data sheet specifically details the space grade version of this product. A more detailed operational description and a complete data sheet for commercial product grades can be found at www.analog.com/AD1672.

- Part Number: The complete part number(s) of this specification follow:

Part Number	Description

AD1672-703F	12-Bit, 3 MSPS, A to D Converter

AD1672-713D 	12-Bit, 3 MSPS, A to D Converter with radiation test

  - Case Outline:

Letter 	Descriptive designator 	Case Outline (Lead Finish per MIL-PRF-38535)

D 	GDIP2-T28 	28-Lead Sidebrazed DIP package

F 	CDFP3-F28 	28-Lead bottom-brazed flatpack

| PIN DESCRIPTION |  |  |  |
| --- | --- | --- | --- |
| Symbol | Pin No. | Type | Name and Function |
| DRCOM | 1 | P | Digital Output Drive Ground |
| BIT 12 | 2 | DO | Data Bit (LSB). |
| Bit 2-11 | 3-12 | DO | Data Bits |
| BIT 1 | 13 | DO | Data Bit (MSB). |
| DRVDD | 14 | P | +5V Digital Output Drive Supply |
| OTR | 15 | DO | Out of Range is Active High on the leading edge of Code 0 or the trailing edge of Code 4096. See Output Data Format Table V. |
| CLOCK | 16 | DI | Sample Clock |
| VDD | 17 | P | +5V Digital Supply |
| DCOM | 18 | P | Digital Ground |
| REFCOM | 19, 24 | P | Analog Ground |
| REFOUT | 20 | AO | 2.5V Reference Output (Decouple with 1F ceramic capacitor to REFCOM). |
| AIN1 | 21 | AI | Analog Input |
| AIN2 | 22 | AI | Analog Input |
| REFIN | 23 | AI | Reference Input |
| NCOMP2 | 25 | AO | Noise Compensation (Decouple with 1F ceramic capacitor to ACOM). |
| NCOMP1 | 26 | AO | Noise Compensation (Decouple with 1F ceramic capacitor to ACOM). |
| ACOM | 27 | P | Analog Ground |
| VCC | 28 | P | +5V Analog Supply |

TYPE:  AI = Analog Input; DI = Digital Input; P = Power

AO = Analog Output; DO = Digital Output

Figure 1 - Terminal connections.

- Absolute Maximum Ratings. (TA = 25°C, unless otherwise noted)

VCC with respect to ACOM	-0.5 to +6.5V

VDD with respect to DCOM	-0.5 to +6.5V

DRVDD with respect to DRCOM	-0.5 to +6.5V

ACOM with respect to DCOM, DRCOM	-0.5 to +0.5V

Clock with respect to DCOM	-0.5 to VDD + 0.5V

Digital Outputs with respect to DCOM	-0.5V to DRVDD + 0.5V

AIN with respect to ACOM	-6.5V to +6.5V

REFIN with respect to ACOM	-0.5V to VCC + 0.5V

Junction Temperature (TJ)	 +150°C

Operating Temperature Range	-55°C to + 125°C

Storage Temperature	-65°C to +150°C

Lead Temperature (10 sec)	 +300°C

Stresses above those listed under "Absolute Maximum Ratings" may cause permanent damage to the device. This is a stress rating only and functional operation of the device at these or any other conditions above those indicated in the operational sections of the specification is not implied. Exposure to absolute maximum ratings for extended periods may effect device reliability.

  - Thermal Characteristics:

Thermal Resistance, Sidebrazed (D) Package

Junction-to-Case (JC) = 28°C/W Max

Junction-to-Ambient (JA) = 70°C/W Max

Thermal Resistance, Bottom brazed (F) Package

Junction-to-Case (JC) = 22°C/W Max

Junction-to-Ambient (JA) = 60°C/W Max

- Electrical Table:

| Table I |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Parameter (see notes at end of table) | Symbol | Conditions 1/ Unless Otherwise Specified | Sub Group | Limit Min | Limit Max | Units |
| Resolution |  | No Missing Codes | 1,2,3 | 12 |  | Bits |
| Supply Current | IVCC |  | 1,2,3 |  | 65 | mA |
|  | IVDD |  |  |  | 2 |  |
|  | IDRVDD |  |  |  | 2 |  |
| Power Dissipation | PD |  | 1,2,3 |  | 363 | mW |
| Power supply rejection 3/ | PSR | VCC = 5.0V ± 0.25V | 1,2,3 | -0.3 | 0.3 | %FSR |
|  |  | VDD = 5.0V ± 0.25V |  | -0.3 | 0.3 |  |
|  |  | VDRDD = 3.0V to 5.25V |  | -0.1 | 0.1 |  |
| ACCURACY |  |  | 1,2,3 |  |  |  |
| Integral Nonlinearity | INL |  |  | -2.5 | 2.5 | LSB |
| Differential Nonlinearity | DNL |  |  | -1 | 1.5 | LSB |
| Offset Zero | VOSE |  |  | -0.75 | 0.75 | %FSR |
| Zero Error 5/ |  |  |  | -0.75 | 0.75 | %FSR |
| Gain Error 2/ |  |  |  | -1.5 | 1.5 | %FSR |
| Internal Voltage Reference 6/ | VO | IOUT = 0.5 mA 4/ | 1,2,3 | 2.475 | 2.525 | V |

| Parameter See notes at end of table | Symbol | Conditions 1/ Unless Otherwise Specified | Sub Group | Limit Min. | Limit Max | Units |
| --- | --- | --- | --- | --- | --- | --- |
| Analog Input |  | 2.5V Range Unipolar | 1,2,3 | 0 | 2.5 | V |
|  |  | 5.0V Range Unipolar |  | 0 | 5 |  |
|  |  | 5.0V Bipolar |  | -2.5 | 2.5 |  |
| Input Resistance |  | 2.5V Input Range 5.0V Input Range | 1,2,3 | 1.5 3 | 2.5 5 | K |
| Output Voltage, High | VOH | IOH= 0.5 mA | 1,2,3 | 2.4 |  | V |
| Output Voltage, Low | VOL | IOL= 1.6 mA | 1,2,3 |  | 0.4 | V |
| Logic Inputs Logic “0” Input Current Logic “1” Input Current | IIH IIL | VIH=VDD VIL=0V | 1,2,3 | -10 | 10 | uA |
| AC Parameters: |  |  |  |  |  |  |
| Parameter See notes at end of table | Symbol | Conditions 1/ Unless Otherwise Specified | Sub Group | Limit Min. | Limit Max | Units |
| Signal to Noise Distortion | S/(N+D) | finput=500KHz | 4 5 6 | 63 59 62 |  | dB |
| Signal to Noise Ratio | SNR | finput=500KHz | 4 5 6 | 66 59 62 |  | dB |
| Total Harmonic Distortion | THD | finput=500KHz | 4,5,6 | -64 |  | dB |
| Spurious Free Dynamic Range | SFDR | finput=500KHz | 4,5,6 | -65 |  | dB |

4.0	Electrical Table: (Cont’d)

NOTES:

1/ VCC = VDD = VDRVDD = +5.0V.

2/ Includes internal reference error.

3/ Change in full scale as a function of the dc supply voltage.

4/ Current available for external loads. External load should not change during conversion.

5/ Bipolar Mode

6/ The AD1672 includes an onboard +2.5 V reference. The reference input pin (REF IN) can be connected to reference output pin (REF OUT) or a standard external +2.5 V reference can be selected to meet specific system requirements. The reference input voltage can be held with the use of a capacitor. To prevent the AD1672’s onboard reference from oscillating when not connected to REF IN, REF OUT must be connected to +5 VVCC. It is possible to connect REF OUT to +5 V due to its output circuit implementation which shuts down the reference

  - Electrical Test Requirements:

| Table II |  |
| --- | --- |
| Test Requirements | Subgroups (in accordance with MIL-PRF-38535, Table III) |
| Interim Electrical Parameters | 1 |
| Final Electrical Parameters | 1, 2, 3, 4, 5, 6 1/ 2/ |
| Group A Test Requirements | 1, 2, 3, 4, 5, 6 |
| Group C end-point electrical parameters | 1 2/ |
| Group D end-point electrical parameters | 1 |
| Group E end-point electrical parameters | 1 |

Table II Notes

1/ PDA applies to Subgroup 1 only. No other subgroups are included in PDA.

2/ See table III for delta parameters.

  - Table III. Life Test/Burn-In test delta limits.

| Table III |  |  |  |
| --- | --- | --- | --- |
| TEST TITLE | ENDPOINT LIMIT | DELTA LIMIT | UNITS |
| ICC VOH VOL | 65 2.4 0.4 | ±6.5 ±0.24 ±0.1 | mA V V |

- Test/Burn-In Circuit:

5.1 	HTRB is not applicable for this drawing.

5.2 	Burn-in is per MIL-STD-883 Method 1015 test condition D.

5.3 	Steady state life test is per MIL-STD-883 Method 1005.

| Rev | Description of Change | Date |
| --- | --- | --- |
| A | Initiate | 4/20/2000 |
| B | Add Flatpack, Add radiation part number, Add timing information on page 3. | 3/22/2001 |
| C | Update web address. | 2/7/2002 |
| D | Change subgroups 4, 5, 6 to 9, 10, 11. Update web address | 1/9/2003 |
| E | Delete Burn-In circuit. | 8/5/2003 |
| F | Update header/footer and add to 1.0 Scope description | 2/19/2008 |
| G | Add Operating Temperature Range to Section 3.0 & Remove (See Figure 2) in Sections 5.2 and 5.3 | 4/4/2008 |
| H | Remove obsolete part numbers and update ASD to ADI Standard | 11/30/2011 |
| I | Corrected typo in section 4.0 Electrical Table. | 11/08/2013 |
| J | Add application note to prevent oscillation | 01/18/2015 |
| K | Simplify ICC Delta limit by listing absolute value vs % of specification. No change in limit | 07/13/2016 |
| L | Corrected copyright date | 07/29/2016 |
| M | Corrected font errors in last release | 8/23/2016 |
| N | Corrected typo in section 4.0 Electrical Table 1 | 11/1/2016 |
| O | SNR and S/(N+D limit Change for Subgroup 5, Add SINAD symbol and all Subgroup 4,5,6 not 9,10,11 | 9/28/2017 |

| © 2016 Analog Devices, Inc. All rights reserved. Trademarks and registered trademarks are the property of their respective companies. Printed in the U.S.A. 11/16 |  |
| --- | --- |