ADPD

UDP (Bi-Directional) CONTROLLER

and DATA TRANSFER TOOL

USER'S GUIDE

ANALOG DEVICES, INC.

www.analog.com

Rev 0.6, February 2017

Table of Contents

1	Introduction	5

2	Introduction- UDP Communication	5

3	UDP Transfer Tool	5

3.1	Raw Data format	8

4	UDP Controller Tool	10

4.1	UDP Controller Tool Testing Procedure	10

4.1.1	Connect	11

4.1.2	Disconnect	11

4.1.3	Read Register	11

4.1.4	Write Register	12

4.1.5	Open View	12

4.1.6	Load Configuration	13

4.1.7	Play	13

4.1.8	Stop	14

4.1.9	RAW Data	14

5	Command Status Summary	14

Table of Figures

Figure 1: UDP Configuration	5

Figure 2: Connection	6

Figure 3:  Enable full UDP Transfer	6

Figure 4: Enable partial UDP Transfer	6

Figure 5: UDP Receiver Tool	6

Figure 6: File name to save raw data	7

Figure 7: Closing the file	7

Figure 8: Sample data	8

Figure 9: UDP Controller Tool	10

Figure 10: Read Register Operation	11

Figure 11: Write Register Operation	12

Figure 12: List of dcfg files available in WaveTool configuration directory	13

List of Tables

Table 1: UDP Data format	8

Table 2: UDP Data count for varies modes	9

Table 3: Command status summary	14

Copyright, Disclaimer & Trademark Statements

Copyright Information

Copyright (c) 2016 Analog Devices, Inc. All Rights Reserved. This software is proprietary and confidential to Analog Devices, Inc. and its licensors. This document may not be reproduced in any form without prior, express written consent from Analog Devices, Inc.

Disclaimer

Analog Devices, Inc. reserves the right to change this product without prior notice. Information furnished by Analog Devices is believed to be accurate and reliable. However, no responsibility is assumed by Analog Devices for its use; nor for any infringement of patents or other rights of third parties which may result from its use. No license is granted by implication or otherwise under the patent rights of Analog Devices, Inc.

Trademark and Service Mark Notice

Analog Devices, the Analog Devices logo, SigmaStudio, Blackfin, SHARC, TigerSHARC, CrossCore, VisualDSP, VisualDSP++, EZ-KIT Lite, EZ-Extender and Collaborative are trademarks and/or registered trademarks “®” of Analog Devices, Inc.

All other brand and product names are trademarks or service marks of their respective owners.

Trademarks and Service Marks must be reproduced according to ADI’s Trademark Usage guidelines. Any licensee wishing to reproduce ADI’s Trademarks and Service Marks must obtain and follow these guidelines for the specific marks to be reproduced.

# Introduction

This document explains about various commands for controlling the WaveTool, receiving the raw data and its data format for various modes over UDP protocol.

The sample applications are developed and validated in Python 2.7.

# Introduction- UDP Communication

The UDP communication protocol used to communicate with the OpenMarket ADPD WaveTool by a Controller and Receiver application running on locally or remotely.

Through the Controller application, we can communicate with the WaveTool to take the following actions via UDP/IP:

  - Connect

  - Disconnect

  - Open View

  - Load Configuration

  - Play

  - Stop

  - Register Read

  - Register Write

  - Raw Data

# UDP Transfer Tool

- Open the WaveTool  and Go to Tools > Options > UDP Configuration. Enter the IP Address of the remote machine where UDP transfer Tool is running(The target board should not be connected in order to edit the UDP configuration fields).

Figure 1: UDP Configuration

- Connect the hardware using UART Bridge on the host machine.

Figure 2: Connection

- Open Graph View and go to Time View tab and check the UDP Transfer check box.

Figure 3:  Enable full UDP Transfer

The full UDP Transfer mode enables UDP communication and the raw data will not be shared to the graph, hence the graph will not be updated in this mode.

Figure 4: Enable partial UDP Transfer

The partial UDP Transfer mode enables UDP communication with graph update, hence in this mode both UDP communication and the graph will be operable state.

- Click on Play button [      ] to send the raw data through UDP protocol.

- Start the UDP Transfer Tool by running the UDPTransferTool.py python file, you should see a window similar to what is shown in Figure 5.

Figure 5: UDP Receiver Tool

- Enter the log file name in UDP Transfer Tool to save the raw data from the WaveTool.

Figure 6: File name to save raw data

- The received packet count is continuously displayed in the tool and the raw data is stored in the log file.

- Press the Escape key to stop receive the data and close the current file.

Figure 7: Closing the file

- Press  Ctrl+C to Quit the UDP Transfer Tool.

## Raw Data format

The stored sample raw data is given below

Figure 8: Sample data

The sample data is transmitted in the form of ASCII Bytes.

Sample Data Format: F0136CFC0000000FE90000105000000EA000000FCC

- First two characters(0xF0) represents Sync,

- Next two characters(0x13) represents Size,

- Next two characters(0x6C) represents Sequence Number,

- Next Four characters(0xFC00) represents Timestamp,

- Remaining characters are representing the hexadecimal channel raw data and it's count lies based on the slot modes of the WaveTool.

| F0 | 13 | 6C | FC00 | 00000FE9 0000105000000EA000000FCC |
| --- | --- | --- | --- | --- |
| Sync (2) | size (2) | Sequence Number (2) | Time Stamp (4) | Raw Data |

Table 1: UDP Data format

| Slot Mode | Data format | Data count |
| --- | --- | --- |
| Slot A Sum 16-Bits Slot B Sum 16-Bits Slot A Sum 32-Bits Slot B Sum 32-Bits Slot A Dig int_1 16-Bits Slot B Dig int_1 16-Bits Slot A Dig int_1 32-Bits Slot B Dig int_1 32-Bits Slot A Proximity | 00000FE9 | 1 |
| Slot A Dig int_2 16-Bits Slot B Dig int_2 16-Bits Slot A Dig int_2 32-Bits Slot B Dig int_2 32-Bits | 00000FE9 00001050 | 2 |
| Slot A 16-Bits Slot B 16-Bits Slot A 32-Bits Slot B 32-Bits | 00000FE9 00001050 00000EA0 00000FCC | 4 |

Table 2: UDP Data count for varies modes

Note: Make sure the WaveTool is opened and available at the running state (with UDP transfer enabled) before using the UDP Transfer Tool.

# UDP Controller Tool

The UDP controller tool controls the WaveTool actions from the same or remote system(PC) in the same network using UDP protocol. The structure and example of each command can be seen in Figure 8.

Figure 9: UDP Controller Tool

## UDP Controller Tool Testing Procedure

Start executing the UDP Controller Tool and enter the IP address of the system where the WaveTool runs with above described configuration .

After enter the corresponding IP address, user can send/receive commands between UDP Controller Tool and WaveTool.

### Connect

Open the ADPD OpenMarket WaveTool application and make sure the UDP Control IP address is assigned correctly in the UDP configuration panel in the option window in Tool menu.

- The target board can be connected with the WaveTool by sending Connect command as 0A 00 to connect the target board with the WaveTool via auto-connect method.

- 0A  Auto-connect command

- 00  Number of Parameters

- The response message format is 99 00 00

- 99  Auto-connect response id

- 00  Command status(refer  Table 3)

- 00  Number of Parameters

### Disconnect

- The target board can be disconnected from WaveTool by sending the Disconnect command as 0B 00

- 0B  Disconnect command

- 00  Number of Parameters

- The response message format is 91 00 00

- 91  Disconnect response id

- 00  Command status(refer Table 3)

- 00  Number of Parameters

### Read Register

- Send the Register Read Command as 00  01 <reg address> to read the register value from the sensor.

For example send as “00 01 01”

    - 00  Register Read Command

    - 01 Number of Parameter

    - 40  Register Address

- The response message format is 80 00 02 <reg value>

- 80  Register Read response id

- 00  Command status(refer  Table 3)

- 02  Number of Parameters

- < reg value >  Register value in given register address.

Figure 10: Read Register Operation

### Write Register

Open the ADPD OpenMarket WaveTool Application and connect the port and make sure the UDP Control IP address is assigned correctly,

- Send the Register Write Command as 01 03 <reg address> <Value>,

For example send as “01 03 01 00 01”

    - 01  Register Write Command

    - 03  Number of Parameter

    - 01  Register Address

    - 00 01  Register Value

- The response message format is 81 00 00

- 81  Register Write response id

- 00  Command status(refer Table 3)

- 00  Number of Parameters

Figure 11: Write Register Operation

### Open View

This command opens the graph view in WaveTool.

- The command format is 0C 00

    - 0C  Open view Command

    - 00  Number of Parameter

- The response message is 98 00 00

    - 98  Open view response id

    - 00  Command status(refer  Table 3)

    - 00  Number of Parameter

### Load Configuration

- Send the Load configuration file Command as 0D 00,

for example send as “0D 00” then it will give the list of available configuration file names as shown below,

Figure 12: List of dcfg files available in WaveTool configuration directory

We can select the particular configuration file using the following command,

Ex., 0D 01 (01/02/03......./07)

based on which configuration we want to load from the above list.

- The response message is 97 00 00

    - 97  Load configuration response ID

    - 00  command status(refer Table 3)

    - 00  Number of Parameter

### Play

- Send the Run Command as 02 00 , for example send as “02 00”

    - 02  Run Command

    - 00  Number of Parameter

- The response message is 82 00 00

    - 82  Run response id

    - 00  Command status(refer Table 3)

    - 00  Number of Parameter

### Stop

- Send the STOP Command as 03 00, for example send as “03 00”

    - 03  Stop Command

    - 00  Number of Parameter

- The response message is 83 00 00

    - 83  Stop response ID

    - 00  Command status(refer Table 3)

    - 00  Number of Parameter

### RAW Data

The RAW data command to control the raw data transmission through UDP protocol.

- Send the RAW Data command as 06 01 <Control parameter>

    - 06  Raw Data Command

    - 01  Number of Parameter

- Control Command:

- 01  Enable RAW Data transmission.

- 00  Disable RAW Data transmission.

- The response message is 86 00 00

    - 86  Raw Data response ID

    - 00  Command status(refer Table 3)

    - 00  Number of Parameter

# Command Status Summary

| Status ID | Status message |
| --- | --- |
| 00 | Success |
| 01 | Failure |
| 04 | Graph view is not opened |
| 05 | Already sensor is running |
| 06 | Already sensor has stopped |
| 09 | Command not applicable |
| 0A | Invalid command |
| 0B | Target board is already connected |
| 0C | Graph view is already opened |
| 0D | Configuration load failed |
| 0E | Invalid parameter |
| 1A | Target board has been already disconnected |
| 1B | Load configuration is failed because of sensor is in running state |
| B0 | Load configuration is failed |
| C0 | Target board is not connected |
| D0 | Invalid selection of configuration file from list |
| E0 | Configuration file is not found |
| F0 | Register Write/Read failed |

Table 3: Command status summary