MECHANICAL DESCRIPTION OF BOARD

MAX20098 Evaluation Kit REV-P2

| Material: | RoHS Compliant FR-4 Laminate material compatible with Lead-Free soldering processes |
| --- | --- |
| Size: | 2.52 x 2.52 inches |
| Thickness: | 0.062” |
| Layers: | 4 |
| Solder Mask: | GREEN LPI SMOBC |
| Legends: | White (clipped all legends from exposed metal) Top and Bottom silk |
| Copper Clad: | Outer layers: 2 OZ (min finished) Inner layers: 1 OZ |

| PLATING | Must be Lead Free & RoHS compliant |
| --- | --- |
| Finish: | Vendor should use the most economical Lead Free & RoHS compliant process available or as specified in PO. Approved Finish: HASL Lead-Free solder Immersion Gold |
| Thru Holes: | 0.001” min |
| Quality: | Manufactured in accordance with IPC-A-600 |

NUMBER OF SURFACE MOUNT PADS:

TOLERENCES:

| Board Dimensions | +/- 0.010” |
| --- | --- |
| Plated-Thru Holes: | +/- 0.003” |
| Pattern-to Pattern | +/- 0.005” |
| Solder Mask to Pattern | +/- 0.005” |
| Legend to Legend | +/- 0.007” |

Drill Schedule is located on the Drill Plot.

Notes:

- AGND and PGND are connected to EP. This is not a mistake.

- Oval pads with two holes are to be manufactured as shown, NOT as a slot.

Justina Banks

Maxim Integrated

531 E. Elliot Road

Chandler, AZ 85225

Phone:	+1 (480) 503-5890

Email:	justina.banks@maximintegrated.com

04/17/2016