MECHANICAL DESCRIPTION OF BOARD

MAX7032 EVALUATION BOARD REV-A

| Material: | Epoxy Glass FR4 |
| --- | --- |
| Size: | 3.400”X 3.400” |
| Thickness: | 0.062” |
| Layers: | 2 |
| Solder Mask: | Green LPI SMOBC |
| Legends: | White (clipped all legends from exposed metal) |
| Copper Clad: | 1 OZ |

VENDOR LOGO & DATE CODE:	Allow only in Ink on bottom side only.

PLATING:

| Thru Holes: | 0.001” min |
| --- | --- |
| HASL | 0.0002” min |

SURFACE MOUNT PADS:			126

TOLERENCES:

| Board Dimensions | +/- 0.010” |
| --- | --- |
| Plated-Thru Holes: | +/- 0.003” |
| Pattern-to Pattern | +/- 0.005” |
| Solder Mask to Pattern | +/- 0.005” |
| Legend to Legend | +/- 0.007” |

Drill Schedule is located on the Drill Plot.

Helen Gan

Maxim Integrated Products

120 San Gabriel Dr.

Sunnyvale CA 94086

(408) 530-6071

Fax (408) 331-1261

2/18/05