Summary of AD7689/82 Datasheet Changes for PCN

- Increased VIO Range from 2.3V  VDD to 1.8V VDD.

- Improved max Conversion Rate for VDD = 2.3 to 2.7V from 190-kSPS to 200-kSPS.

- Improved Gain Error min/max spec from +30 LSB’s to +8 LSB’s.

- Pushed out Gain Error Match (across channels) spec from +2 LSB’s to + 4 LSB’s.

- Added an Offset Error min/max spec of +8 LSB’s (previously only typ’ed).

- Pushed out Offset Error Match (across channels) spec from +2 LSB’s to + 4 LSB’s.

- Pushed out max Power Dissipation specs on the following conditions:

  - VDD = 5V, 250kSPS:

15.9mW max pushed out to 18mW max.

  - VDD = 5V, 250kSPS, w/ Internal Ref:

19.2mW max pushed out to 21mW max.

- Timing Specs Table 3 (on old d/s) had conditions:

VDD = 4.5V to 5.5V, VIO = 2.3V to VDD, Tmin to Tmax

is now called Table 4 with the following conditions:

VDD = 4.5V to 5.5V, VIO = 1.8V to VDD, Tmin to Tmax

Changes in this Table include:

  - tDATA max changing from 1.4 us to 1.2 us.

  - tSCK min changing from 15 ns to (tDSDO + 2) ns.

  - tSCKL min changing from 7 ns to 11 ns.

  - tSCKH min changing from 7 ns to 11 ns.

  - tSDIN min changing from 4 ns to 5 ns.

  - tHDIN min changing from 4 ns to 5 ns.

- Timing Specs Table 4 (on old d/s) had conditions:

VDD = 2.5V to 4.5V, VIO = 2.3V to VDD, Tmin to Tmax

is now called Table 5 with the following conditions:

VDD = 2.3V to 4.5V, VIO = 1.8V to VDD, Tmin to Tmax

Changes in this Table include:

  - tCONV max for “VDD=2.3V to 2.7V” changing from 3.4 us to 3.2 us .

  - tCYC min for “VDD=2.3V to 2.7V” changing from 5.2 us to 5 us.

  - tSCK min changing from 25 ns to (tDSDO + 2) ns.

  - tDSDO max for “VIO Above 2.3V” changing from 37 ns to 38 ns.

  - tDSDO max spec was added for case “VIO Above 1.8V” with spec of 48 ns.

  - tEN max spec was added for case “VIO Above 1.8V” with spec of 45 ns.