Document No.		:	18-081443-01-A

Title			:	EVAL-CN0586-ARDZ Customer Evaluation Board Test Procedure

| REVISION HISTORY |  |  |  |  |
| --- | --- | --- | --- | --- |
| Revision | ECR # | Description of Change | Date | Author |
| A |  | Initial Release | 2/27/24 | Ian Vincent Andal |

| Required Approvers |  |
| --- | --- |
| Approver Roles | Approver Names |
| Apps Engineer | Ian Vincent Andal |

Contents

Hardware List	3

Software and Driver List	3

Setup	3

Software Download and Installation	4

Hardware Setup	5

SDP EEPROM Programmer Procedures	6

Flashing the Firmware	10

Evaluation Board Test Procedure	10

Hardware Functionality Check	10

System Functionality Check	11

# Hardware List

- Computer (OS: Windows 10 64 bit)

- EVAL-CN0586-ARDZ

- EVAL-SDP-CK1Z

- Dual Tracking Power Supply

- DMM (300V tolerance)

- Oscilloscope and Probe (300V tolerance)

# Software and Driver List

- cn0586_ad5754r_test_fw.bin

- SDPEEPROMProgrammerInstall_1.0.110.0.exe

- CN0586.dat

- CN0586_test1.exe

- CN0586_test2.exe

# Setup

Figure 1 Board Image*

Author Note: The board images used in this version of the document are from a previous revision of the PCB. Some components may differ with the current PCB build but should serve the intended purpose as a guide in the test setups.

## Software Download and Installation

- Before connecting the evaluation boards, download and install the required software and files by following the proceeding instructions.

- Access the FTP server using the provided FTP details:

    - Username: evalcd_readaccess

    - Password: !Subcon123

    - Account: evalcd_readaccess

    - FTP Server/Host: ftp.analog.com

    - Logon Type: Account

    - FTP Link or Directory: /users/evalcd_readaccess/evalcd_read/EVAL-CN0586

Figure 2 FTP Login Details

- Download and install the software listed in the "Software List" from the FTP link provided above to your designated local folder (these are all one-time software installations).

## Hardware Setup

- Configure the jumpers/links as stated below:

    - JP1, JP2, JP3, JP4, JP5 & JP6 – pos A

    - JP7, JP8, JP9, JP10 – pos A (soldered)

    - JP11, JP12 – short

    - S1 – pos B

- Connect the EVAL-CN0586-ARDZ to the Arduino connector of SDP-K1 (EVAL-SDP-CK1Z) as shown in .

- Connect dual tracking power supply, +/-15V, 100mA, to P1. Do not turn on power yet.

- Attach the USB cable from the SDP-K1 to the PC. LED1 (red) on the SDP-K1 will blink red when +/-15V is still powered off. To clear red led, hit reset, or reattach the USB cable.

Figure 3 EVB, SDP-K1 and Laptop Connection

# SDP EEPROM Programmer Procedures

- Open the SDP EEPROM Programmer in Start Menu > SDP EEPROM Programmer (.NET). The main window shown in Figure 4 would be launched.

Figure 4. SDP EEPROM Programmer Window

- On the bottom right portion of the window click on Connect. The main window will change the display as shown in Figure 5.

  - If a prompt appears to reprogram the SDP-K1, click on yes.

Figure 5. SDP-K1-connected Window

- Select the "Load EEPROM content from file" option and navigate to the local folder where the CN0586.dat file is located and then select it. The main window will update to reflect the contents of the file. Please verify that the “EEPROM address” and other entries on the forms match those displayed in Figure 6.

Figure 6 EEPROM Content Load From File Window

- Click on the "Write EEPROM" button located in the center of the window. Wait for the pop-up window to appear, indicating the completion of the EEPROM write process, as shown in Figure 7. Click on the "OK" button to proceed.

- Click the "Disconnect" button situated at the bottom right corner of the window. Finally, close the SDP EEPROM Programmer window to conclude the procedure.

Figure 7 Write to EEPROM Complete Pop-up Window

# Flashing the Firmware

- Do this after the EEPROM Programmer Procedures.

- Connect SDP-K1 to PC via USB cable.

- Locate the cn0586_ad5754r_test_fw.bin Right click and send to SDP-K1 (X:) drive and wait for copying to finish. See Figure 8 for example.

Validation: If you connect the SDP-K1 to the PC without powering-up the +/-15V supplies, or without connecting the EVAL-CN0586-ARDZ, the red LED (LED1) should start blinking. This means the firmware has been successfully flashed.

Figure 8 Flashing the Firmware to the SDP-K1

# Evaluation Board Test Procedure

## Hardware Functionality Check

- Setup the following jumper settings. Unless otherwise noted, jumpers are set in default.

  - JP5, JP6 – Open

- Turn on dual tracking supply +/-15V, 100mA.

- Visual check on LEDs: DS1, DS2, DS3 = On, DS4 = Off

- Probe the test points shown in the table below. Figure 9 shows the test points location.

- Turn off power supplies. Set S1 switch to pos A.

- Turn on power supplies. Continue to probe.

- Turn off power supplies then return all jumpers and switch to default positions.

| Probe point (referred to TP_GNDx) | Expected | Actual | Pass/Fail |
| --- | --- | --- | --- |
| TP_REF | 2.5V, +/-50mV |  |  |
| JP2.1 (JP2 pin 1) | 3.3V, +/-100mV |  |  |
| JP5.2 | 110V, +/-5V |  |  |
| JP5.4 | 15V, +/-100mV |  |  |
| JP6.2 | -110V, +/-5V |  |  |
| JP6.4 | -15V, +/-100mV |  |  |
| Hold – Refer to step 5&6 |  |  |  |
| JP5.2 | 200V, +/-5V |  |  |
| JP6.2 | 0V, +/-1V |  |  |

Figure 9 Probe Locations 1

## System Functionality Check

- Connect the setup as in Hardware Setup. Turn on +/-15V supplies.

- If the red LED is blinking on SDP-K1 after turning on the supplies, press the reset button or reconnect the USB cable.

- Run the CN0586_test1.exe

- Probe the test points shown in the table below. Figure 10 shows the test points location.

| Probe point (referred to TP_GNDx) | Expected | Actual | Pass/Fail |
| --- | --- | --- | --- |
| TP_A | 0V |  |  |
| TP_B | 0V |  |  |
| TP_C | 3.75V +/-10mV |  |  |
| TP_D | 0V |  |  |

Figure 10 Probe Locations 2

- Connect oscilloscope probe to VOUT_HV (ground to TP_GNDx)

- Run CN0586_test2.exe

- Output waveform sample shown in Figure 9. Expected Vpk-pk = 200V, Frequency = 1kHz

- Power off power supplies.

Figure 11 Sample Waveform at HVOUT