MAXREFDES36# BOM

| Designation | Qty | Description |
| --- | --- | --- |
| C1 | 1 | CAP CER 1UF 50V 10% X7R (0603) Taiyo Yuden UMK107AB7105KA-T |
| C3,C12, C24 | 3 | CAP CER 0.1UF 50V 10% X7R (0402) Taiyo Yuden UMK105B7104KV-FR |
| C4B,C5,C7,C10, C21,C33 | 6 | CAP CER 0.1UF 16V 10% X5R (0201) TDK C0603X5R1C104K030BC |
| C8,C9 | 2 | CAP CER 15PF 50V 5% NP0 (0201) TDK C0603C0G1H150J030BA |
| C4A,C6 | 2 | CAP CER 1UF 6.3V 20% X5R (0201) TDKC0603X5R0J105M030BC |
| C11,C23 | 2 | 10uF 20%, 50V X5R ceramic capacitor (1210) TAIYO YUDEN UMK325BJ106MM |
| C22, C34 | 2 | 4.7uF 10%, 10V, X7R ceramic capacitor (0805) MURATA GRM21BR71A475K |
| CIN | 1 | CAP CER 1UF 100V 10% X7S (0805) TDK C2012X7S2A105K |
| COUT | 1 | CAP CER 4.7UF 10V 10% X7S (0805) Murata GCM21BC71A475KA73# |
| CN1 | 1 | M12-A 4-pin Male connector (M12_Edge) Binder 09-0431-212-04 |
| CN2 | 1 | 8-pin Female connector (2x4x1.27) Samtec CLP-104-02-G-D |
| D17 | 1 | Yellow 1mA Led, (0402) Rohm SML-P12YTT86 |
| D18, D20, D21 | 3 | Red 1mA Led, (0402) Rohm SML-P12UTT86 |
| D19, D1-D16 | 17 | Green 1mA Led, (0402) Rohm SML-P12PTT86 |
| D22 | 1 | 36V TVS diode On Semiconductor 1SMB36AT3G |
| D24, D25 | 2 | 36V TVS Diode with 55V clamp (3 SOT23) Semtech SDC36C.TCT |
| L1 | 1 | 100uH Inductor COILCRAFT LPS3015-104ML |
| R0A, R11, RP1, RP2 | 4 | 0 Ohm resistor 50V (0402) 0.125W Vishay/Dale CRCW04020000Z0EDHP |
| RV3, RVP | 2 | 10 Ohm 5% resistor 50V (0402) any |
| R1 | 1 | 1 Ohm 1% resistor 50V (0402) 0.125W / 353mA max Vishay/Dale CRCW04021R00FKEDHP |
| R2, R3 | 2 | 20k Ohm 5% resistor (0603) any |
| R8, R23, R34 | 3 | 1.5k Ohm 5% resistor (0402) 0.1W any |
| R7 | 1 | 10k Ohm 5% resistor (0201) 0.05W / 2.2mA max |
| R9 | 1 | 499k Ohm 1% resistor (0201) 0.05W |
| R10 | 1 | 95.3k Ohm 1% resistor (0201) 0.05W |
| R12 | 1 | 69.8k Ohm 1% resistor (0201) 0.05W |
| R13 | 1 | 150 ohm 1%, 1W MELF resistor (0207) Vishay MMB02070C1500FB200 |
| R14-R21, R25-R32 | 16 | 2.2k ohm 1%, 0.4W MELF resistors (0204) Vishay MMA02040C2201FB300 |
| R22, R33 | 2 | 15k Ohm 5% resistor (0603) any |
| R35, R38, R40, R41, R44, R46 | 6 | 0 Ohm resistor (0402) any |
| TB1 | 1 | Grey 4 port Wire Push-In Terminal Block Wago 250-404 |
| TB2, TB3, TB4 | 3 | Grey 16 port Wire Push-In Terminal Block Wago 250-416 |
| U1 | 1 | IO Link Device Transceiver (25 WLP) Maxim MAX14821EWA+ |
| U2 | 1 | Low Power uC (25 WFLGA) Renesas R5F10E8EALA#U0 |
| U3 | 1 | 60V Ultra Small Step-Down Converter (10 TDFN-EP) Maxim MAX17552ATB+ |
| U4, U5 | 2 | Industrial, Octal, Digital Input Translator/Serializer (28 TSSOP-EP) MAX31913AUI+ |
| Y1 | 1 | 18.432MHz Crystal (4 SMD) Mercury X22-18.432-12-30/30/4085 |
| R36, R37, R39, R42, R43, R45 | 0 | Not installed. 0 Ohm resistor (0402) any |
| C2 | 0 | Not Installed. CAP CER 0.1UF 50V 10% X7R (0402) Taiyo Yuden UMK105B7104KV-FR |
| C2B | 0 | Not installed. CAP CER 0.1UF 50V 10% X7R (0402) Taiyo Yuden UMK105B7104KV-FR |
| CV5 | 0 | Not installled. CAP CER 1UF 10V 10% X5R (0402) TDK C1005X5R1A105K |
| C13-C20, C25-C32 | 16 | Not installed, ceramic capacitor (0603) |
| R0B | 0 | Not installed. 0 Ohm resistor 50V (0402) 0.125W Vishay/Dale CRCW04020000Z0EDHP |
| RV5 | 0 | Not installed .10 Ohm 5% resistor 50V (0402) |
| TP1 | 0 | Not installed .Test Point |
| CASE1 | 1 | DIN Rail PCB Holder Base Element 11.25mm Width OKW Electronics B6720330 |
| CASE2 | 2 | DIN Rail PCB Holder End Section with Foot 22.5mm Width OKW Electronics B6720336 |
| PCB | 1 | PC board: San Francisco (MAXREFDES36#) IO Link 16 Channel 24V Digital Input |

All components not listed above are not installed.