New Reference Design (RD) Definition Template

The Ten Requirements For Every RD

Strategic Definition = green

Engineering Definition = teal

Definition done by: David Andeen and Ted Salazar

Please ensure all Maxim ICs are introduced or WILL be introduced without any known issues within the quarter before starting the project.

Strategic Definition = green

- Quick Reference Design Description:

The low power IO-Link RTD temperature sensor features a high accuracy temperature input that accommodates a wide range of RTD inputs and a wide input temperature range.  The temp sensor performs accurate temperature conversions to 15 bit resolution with multiple fault detection features.  Data gets communicated back to an IO-Link master using the IO-Link communication protocol.  This reference design consumes an extremely low amount of power by utilizing a wide input voltage DC/DC step down converter.  This reference design continues the IO-Link platform for multiple sensor types, equipped with the IO-Link interface.

- Market segment?

Industrial Control and Automation - Sensors

- What is the application and end equipment?

Industrial sensors

- Maxim Advantage /differentiator - What problem does it need to solve? Why is this a better solution than one of the competition’s or something we already have?

This design features a resistance to digital temperature sensor which can also detect multiple faults.  The architecture minimizes space and eliminates the need for additional components.  The IO-Link communication protocol enables quick sensor configuration and reduces cabling while featuring a robust, medium speed, communication protocol with power, enabling higher powered sensors than a 4-20mA loop.  IO-Link provides low cost sensing, reduced wiring, and enhanced communication (including items like diagnostics).

Maxim’s differentiators include:

  - Fast (21msec) and accurate (0.5°C) temperature sensing using multiple RTD inputs

  - Wide input temperature range

  - Fault detection for open and shorted RTD elements

  - Ultra low power, efficient design with industrial DC/DC.

  - High temperature warning and shutdown

  - Reverse polarity and short circuit protection

- Block Diagram  – Indicating key Maxim IC part numbers required: Please also indicate the required power rails in the system.

Additional requirements:

    - IO-Link style connector

    - System may not include an LCD display with a temperature readout - few pins available in the package to drive an LCD; however we may want to go with an SPI LCD and share the SPI port between the MAX31865 and the SPI LCD. P20 will be the CS pin for the SPI LCD.

    - Recommended – a barrel style form factor, similar to the image below

    - The temperature sensing element will not be provided

    - An M12 connection shall be provided to connect the temperature sensing element

- Business potential -  Ideally, what is the potential revenue from the solution

2016 TAM is $1648M

Potential FY16 revenue from the product lines ~$15M

- Who are the customers who want/need this? How many? Who are they? Size?

Customers – Large multinational, industrial process control and sensor companies including:

    - Festo

    - Sick

    - IFM

    - Turck

    - Balluff

    - Omron

Additional, small customers developing industrial control systems and/or sensors are also considered customers.

Engineering ZBB = teal

- Reference Design Orderable Part Number & Marketing Name: (Please also indicate RD Class (Full Ref Design or Subsystem), Form factor (Single Pmod, Dual Pmod, FMC, etc) and Platform (ZedBoard, LX9, RL78G13, RL78G14, RX63N, Arduino etc):

Part number = MAXREFEDES42#

Marketing Name:   Mojave

RD Class = Full Reference Design; Complete sensor/hub including form factor

Form Factor = Sensor

Platform = None; Renesas RL78 microcontroller

- Customer Applications specifications - what specs do the customers need for the application. What are the “must have” reference design specification deliverables, and what are the “would like to have” reference design specification deliverables.

| Customer Specs |  |
| --- | --- |
| Analog | - IO-Link specification V1.0 and V1.1 Physical Layer compliant - Reverse-Polarity Protection and Short Circuit protection on all 24V inputs and outputs. - 24V in compliance with IO-Link - M12 connection for temperature sensing element - 0.5°C accuracy |
| Digital | - IO-Link slave/compliance |
| FW | - Non OS Embedded controller drivers provided for system solution - Simple method for device configuration for 2, 3 and 4 wire connections |

| Deliverables |  |
| --- | --- |
| HW | - Schematic - BOM - PCB Layout Files - Purchasable Assembled Board |
| FW | - Non OS Embedded Controller Driver(s) - Microcontroller Configuration Files - IO-Link communication stack – Customer must purchase - IODD file (IO-Link Device Description) – identifies the sensor to the IO-Link master - Must use Maxim Vendor ID = 478 - Must use Maxim Device ID = 5 - Automatic sample mode using the DRDY line will not be supported because there are not enough IOs to support it. Single shot sampling mode will be supported instead. |

- How will the specs be verified? Is the test equipment needed for verification available or will it have to be rented?

The board will be tested for IO-Link compliance by an existing IO-Link master compliance verification tool as shown below.

Functional test shall be performed.

Test data shall include a plot of temperature measurement error over a range of 150°C (endpoints TBD based on test system availability) using a 4 wire probe connection to a PT1000 sensor, demonstrating accuracy of 0.5°C or better over the example temperature range.

References:

RL78_IO-Link_SBD 7-9-13.pdf – to be inserted in system files

Existing product reference:

IFM IO-Link temp sensor, TR7432

http://www.ifm.com/products/uk/ds/TR7432.htm

Competitive reference:

Analog Devices CN0075 http://www.analog.com/en/circuits-from-the-lab/CN0075/vc.html

Analog Devices CN0287 http://www.analog.com/en/circuits-from-the-lab/cn0287/vc.html