/***************************************************************************/

/* DEMO1388.c                                                              */

/***************************************************************************/

/* #pragma code symbols debug */

#include <stdio.h>		/* Prototypes for I/O functions     */

#include <DS5000.h>		/* Register declarations for DS5000 */

/***************************** Defines ********************************/

#define	ACK	0

#define	NACK	1

#define	ADDRTC	0xd0	/* I2C addresses */

/************************* bit definitions ****************************/

sbit	scl = P0^0;        	/* I2C pin definitions */

sbit	sda = P0^1;

sbit	RSTb = P3^2;

void	start();

void	stop();

void	i2cwrite(unsigned char d);

uchar	i2cread(uchar);

void	readbyte();

void	writebyte();

void	initialize();

void	disp_clk_regs(uchar);

void	eeprom_wrt(uchar, uchar);

void	eepromread();

void	eepromwrite(uchar);

/************************* global variables ***************************/

uchar	msec, sec, min, hr, dy, dt, mn, yr;

void start()	/* ----------------------------------------------- */

{

sda = 1;  scl = 1;	/* Initiate start condition */

sda = 0;

}

void stop()		/* ----------------------------------------------- */

{

sda = 0; scl = 1; scl = 1; sda = 1;	/* Initiate stop condition */

}

void i2cwrite(uchar d)		/* ----------------------------- */

{

uchar i;

scl = 0;

for (i = 1; i <= 8; i++)

{

sda = (d >> 7);

scl = 1;

d = d << 1;	/* increase scl high time */

scl = 0;

}

sda = 1;	/* Release the sda line */

scl = 1;

if(sda) printf("Ack bit missing\n");

scl = 0;

}

uchar	wr_ack(uchar d)	/* --------- write slave address, check for ack -------- */

{

uchar i;

start();

scl = 0;

for (i = 0;i < 8; i++)

{

sda = (d >> 7);

scl = 1;

d = d << 1;	/* put here to increase scl high time */

scl = 0;

}

sda = 1;	/* Release the sda line */

scl = 1;

i = sda;	/* get sda status, ack or nack */

scl = 0;

stop();

return(i);

}

uchar	wr_ret(uchar d)	/* --------- write slave address, check for ack -------- */

{

uchar i;

scl = 0;

for (i = 0;i < 8; i++)

{

sda = (d >> 7);

scl = 1;

d = d << 1;	/* put here to increase scl high time */

scl = 0;

}

sda = 1;	/* Release the sda line */

scl = 0;

scl = 1;

i = sda;	/* get sda status, ack or nack */

scl = 0;

return(i);

}

uchar i2cread(uchar b)	/* ----------------------------------- */

{

uchar i, d;

sda = 1;             /* Let go of sda line */

scl = 0;

for (i = 1; i <= 8; i++)	/* read the msb first */

{

scl = 1;

d = d << 1;

d = d | sda;

scl = 0;

}

sda = b;          /* 1 = NACK, 0 = ACK */

scl = 1;	scl = 1;

scl = 1;	scl = 1;

scl = 0;

sda = 1;          /* Release the sda line */

return d;

}

void writebyte()	/* ------- write a byte to block 0 --------- */

{

uchar	Add, Data;

printf("\nAddress (hex): ");	/* Get Address */

scanf("%bx", &Add);

printf("DATA (hex): ");

scanf("%bx", &Data);	/* and data */

start();

i2cwrite(ADDRTC);

i2cwrite(Add);

i2cwrite(Data);

stop();

}

void readbyte()	/* ---------read a byte from block 0 ---------- */

{

uchar	Add;

printf("\nAddress (hex): ");	/* Get Address */

scanf("%bx", &Add);

start();

i2cwrite(ADDRTC);

i2cwrite(Add);

start();

i2cwrite(ADDRTC | 1);

printf("%2bx", i2cread(NACK) );

stop();

}

void initialize()	/* --- initialize time & date from user entries --- */

/* Note: NO error checking is done on the user entries! */

{

start();		/* The following Enables the Oscillator */

i2cwrite(ADDRTC);	/* address the part to write */

i2cwrite(0x0b);	/* position the address pointer to 1 */

i2cwrite(0);		/* clear the OSF, WF bits */

i2cwrite(0);		/* enable osc, disable WDE, WD/RST */

stop();

printf("\nEnter the year (0-99): ");

scanf("%bx", &yr);

printf("Enter the month (1-12): ");

scanf("%bx", &mn);

printf("Enter the date (1-31): ");

scanf("%bx", &dt);

printf("Enter the day (1-7): ");

scanf("%bx", &dy);

printf("Enter the hour (1-23): ");

scanf("%bx", &hr);

hr = hr & 0x3f;	/* force clock to 24 hour mode */

printf("Enter the minute (0-59): ");

scanf("%bx", &min);

printf("Enter the second (0-59): ");

scanf("%bx", &sec);

start();

i2cwrite(ADDRTC);	/* write slave address, write 1388 */

i2cwrite(0);		/* write register address, 1st clock register */

i2cwrite(0);		/* default for msec register */

i2cwrite(sec);

i2cwrite(min);

i2cwrite(hr);

i2cwrite(dy);

i2cwrite(dt);

i2cwrite(mn);

i2cwrite(yr);

stop();

}

void	disp_clk_regs(uchar prv_sec)	/* ----- time & date display ----- */

{

uchar	mil, pm;

printf("\n    Yr/Mn/Dt Dy Hr:Mn:Sc");

while(!RI)	/* Read & Display Clock Registers */

{

start();

i2cwrite(ADDRTC);	/* slave address, write 1388 */

i2cwrite(0);		/* write register address, 1st clock register */

start();

i2cwrite(ADDRTC | 1);	/* slave address + read bit, read 1388 */

msec = i2cread(ACK);	/* starts w/last address stored in register pointer */

sec = i2cread(ACK);

min = i2cread(ACK);

hr = i2cread(ACK);

dy = i2cread(ACK);

dt = i2cread(ACK);

mn = i2cread(ACK);

yr  = i2cread(NACK);

stop();

if(hr & 0x40)

mil = 0;

else

mil = 1;

if(sec != prv_sec)	/* display every time seconds change */

{

if(mil)

{

printf("\nDUT %02bX/%02bX/%02bX %02bX", yr, mn, dt, dy);

printf(" %02bX:%02bX:%02bX.%02bx", hr, min, sec, msec);

}

else

{

if(hr & 0x20)

pm = 'P';

else

pm = 'A';

hr &= 0x1f;	/* strip mode and am/pm bits */

printf("\nDUT %02bx/%02bx/%02bx %02bx", yr, (mn & 0x1f), dt, dy);

printf(" %02bx:%02bx:%02bx.%02bx %cM", (hr & 0x1f), min, sec, msec, pm);

}

}

if(prv_sec == 0xfe)	return;

prv_sec = sec;

}

RI = 0;  /* Swallow keypress to exit loop */

}

void	eeprom_wrt(uchar blk, uchar dat)	/* --- write single block of eeprom data --- */

{

uchar	inc;

start();

i2cwrite(ADDRTC + blk);	/* write slave address, write 1388 */

i2cwrite(0);			/* starting address */

for(inc = 0; inc < 8; inc++)

i2cwrite(dat);

stop();

do

{

dat = wr_ack(ADDRTC + blk);

}	while(dat);	/* wait for ack; indicates EEPROM write is finished */

}

void	eeprom_byte_wrt()	/* --- write single byte of eeprom data --- */

{

uchar	inc;

start();

i2cwrite(ADDRTC + 2);	/* write slave address, write 1388 */

i2cwrite(4);			/* starting address */

i2cwrite(0x11);

stop();

do

{

inc = wr_ack(ADDRTC + 2);

}	while(inc);	/* wait for ack; indicates EEPROM write is finished */

}

void	eepromwrite(uchar Data)	/* -------- fill EEPROM with fixed data -------- */

{

uchar blk, j, k, m;

for(blk = 2; blk < 5; blk <<= 1)

{

for(j = 0; j < 32; j++)	/* write until the pointer wraps around */

{

start();

i2cwrite(ADDRTC + blk);	/* write slave address, write 1388 */

i2cwrite(j * 8);	/* write register address, 1st address in block */

for(k = 0; k < 8; k++)

{

i2cwrite(Data);

}

stop();

do

{

start();

k = wr_ret(ADDRTC + blk);	/* access an EEPROM block */

stop();

}	while(k);	/* wait for ack; indicates EEPROM write is finished */

}

}

}

void eepromread()		/* ----------- read the EEPROM in one shot --------- */

{

uchar blk, j, k;

for(blk = 2; blk < 5; blk <<= 1)

{

start();

i2cwrite(ADDRTC + blk);	/* write slave address, write 1388 */

i2cwrite(0);		/* word address (1st RAM location) */

start();

i2cwrite(ADDRTC + blk + 1);	/* write slave address, read 1388 */

for(j = 0; j < 255; j++)

{

if(!(j % 16) )	printf("\n%02bx ", j);

printf("%02bX ", i2cread(ACK) );

}

printf("%02bX\n", i2cread(NACK) );	/* last byte must NACK */

stop();

}

}

main (void)	/* ----------------------------------------------------- */

{

uchar	M, M1;

RSTb = 1;	/* set up for read, weak pull-up */

while (1)

{

printf("\nDS1388 build date: %s\n", __DATE__);

printf("CR Clock Read  CI Init RTC\n");

printf("BR Byte Read   BW Write Byte\n");

printf("ER EEPROM Read EW EEPROM Write\n");

printf("EB EEPROM Block wr\n");

printf("Enter Menu Selection: ");

M = _getkey();

switch(M)

{

case 'B':

case 'b':

printf("\nRead or Write: ");

M1 = _getkey();

switch(M1)

{

case 'R':

case 'r':	readbyte(); 	break;

case 'W':

case 'w':	writebyte(); 	break;

}

break;

case 'C':

case 'c':

printf("\rEnter Clock Routine to run:C");

M1 = _getkey();

switch(M1)

{

case 'I':

case 'i':	initialize();	break;

case 'R':

case 'r':	disp_clk_regs(0x99);	break;

}

break;

case 'E':

case 'e':

printf("\rEnter Ram Routine to run:R");

M1 = _getkey();

switch(M1)

{

case 'B':

case 'b':	printf("\nEnter block (2 or 4): ");

scanf("%bx", &M);

printf("Enter data (hex): ");

scanf("%bx", &M1);

eeprom_wrt(M, M1);

M = 'E';	M1 = 'b';	break;

case 'R':

case 'r':	eepromread();	break;

case 'W':

case 'w':	printf("\nEnter the data to write: ");

scanf("%bx", &M1);

eepromwrite(M1);	break;

case 'X':

case 'x':	eeprom_byte_wrt();	break;

}

break;

}

}